NFPP Sodium-Ion Cell Optimization and Model-Based Battery Management System Design for Grid-Resilient Energy Storage Systems

A PROPOSAL

BY

PAUL CHINONSO IREM

U19EE1013

MARCH 2026

## Abstract

This study presents a cost-driven electrochemical optimization framework for sodium-ion battery energy storage systems (ESS) targeted at deployment in underdeveloped electrical grids characterized by instability, intermittent supply, and infrastructure constraints. The baseline system is built on a sodium iron pyrophosphate (NFPP) cathode paired with a hard carbon anode, aluminum current collector, and a carbonate-based electrolyte containing sodium hexafluorophosphate dissolved in mixed organic carbonate solvents. While NFPP-based sodium-ion batteries offer advantages in elemental abundance, safety, and supply chain resilience, their current implementations remain cost-inefficient relative to lithium iron phosphate (LFP) systems, necessitating coordinated optimization across materials and cell design.

The primary objective of this work is to identify NFPP-derived sodium-ion cell configurations that minimize production cost while improving electrochemical efficiency. The study focuses on cell-level optimization, while battery management system (BMS) design is treated separately for operational control in unstable grid environments.

The computational pipeline first models the baseline sodium-ion cell using a Doyle–Fuller–Newman (DFN) electrochemical framework coupled with thermal PDE heat equations to resolve internal temperature evolution T(x,t), electrochemical heat generation, electrolyte concentration gradients, particle stoichiometry evolution, overpotential formation, and terminal voltage dynamics under discharge operation.

Sensitivity analysis is then performed on:  geometry and structural electrode parameters using PyBaMM geometry modules, and interface kinetics governing electrode–electrolyte coupling behaviour.

The optimization problem is subsequently formulated as a constrained geometric projection problem to identify physically feasible parameter configurations satisfying electrochemical, thermal, structural, and manufacturing constraints.

Selected configurations undergo reduced-order DFN-consistent validation, evaluating discharge response, SOC/HOC evolution, and thermal PDE-driven internal temperature distribution T(x,t)under peak loading and transient current conditions.

Following validation, a battery management system (BMS) is developed independently, using adaptive and model-based control strategies to regulate state-of-charge limits, current profiles, and power delivery in response to internal states and external disturbances, enabling stable operation under grid-induced stress, supply intermittency, and load variability typical of underdeveloped electrical networks.

This work presents a computationally integrated framework for sodium-ion battery optimization, combining cost-aware materials optimization, physics-based electrochemical validation, and control-oriented battery management system design. The proposed approach offers a pathway toward economically viable and operationally robust sodium-ion energy storage solutions for underdeveloped electrical grids.

## Keywords

Sodium-ion battery; NFPP cathode; Hard carbon anode; PyBaMM; FEniCSx; DFN electrochemical modeling; Thermoelastic modeling; Thermal PDEs; Cell-level optimization; Electrolyte engineering; BMS control; Stationary energy storage; Grid-constrained operation.

# Methodology

# Base Cell Model (Literature-Aligned NFPP Sodium-Ion Twin System)

# 1. Electrochemical Core (DFN-Compatible Reaction)

# The sodium iron pyrophosphate (NFPP) cathode operates via reversible sodium intercalation:

# Na₂FePO₄P₂O₇ ⇌ NaₓFePO₄P₂O₇ + (2 − x)Na⁺ + (2 − x)e⁻

# Theoretical specific capacity: ~95–100 mAh g⁻¹, consistent with reported polyanionic NFPP sodium-ion cathode systems used in pouch-scale prototypes.

# 2. Cathode Electrode Architecture (Composite Design)

# NFPP cathodes in practical sodium-ion full cells follow a carbon–binder–domain composite structure processed using N-methyl-2-pyrrolidone (NMP)-based slurry casting.

# Fixed composition:

- Sodium iron pyrophosphate (NFPP) active material: 85 wt%

- Conductive carbon additive (carbon black / acetylene black): 8 wt%

- Binder: polyvinylidene fluoride (PVDF): 7 wt%

# This structure reflects standard aluminum current collector-based cathodes used in sodium-ion pouch cells with high-density electrode compaction.

# 3. Anode Design (Hard Carbon System)

# Hard carbon anodes are implemented as disordered carbon networks with nanopore and turbostratic domains enabling sodium storage through adsorption, intercalation, and pore filling mechanisms.

# Fixed formulation:

- Hard carbon active material: 88 wt%

- Conductive carbon additive: 6 wt%

- Binder: polyvinylidene fluoride (PVDF): 6 wt%

# Practical specific capacity: 250–300 mAh g⁻¹, consistent with full-cell hard carbon sodium storage behavior.

# 4. Electrolyte System (Carbonate-Based Sodium Salt System)

# The electrolyte follows a standard sodium-ion full-cell carbonate formulation:

- Sodium hexafluorophosphate (NaPF₆): 1.0 molar concentration

- Sodium difluoro(oxalato)borate (NaDFOB): 0.2 molar concentration

- Solvent system: ethylene carbonate and propylene carbonate in 1:1 volumetric ratio

- Ionic conductivity: ~10 mS cm⁻¹ at 25°C

# 5. Electrolyte Additive System (Interphase Engineering)

# Interfacial stability is controlled using electrolyte additives that regulate both solid electrolyte interphase and cathode electrolyte interphase formation:

- Fluoroethylene carbonate (FEC): 3 wt%
→ promotes stable solid electrolyte interphase (SEI) formation on the hard carbon anode

- Vinylene carbonate (VC): 2 wt%
→ enhances SEI uniformity and suppresses continuous electrolyte decomposition

- Sodium difluoro(oxalato)borate (NaDFOB): functions as both co-salt and cathode electrolyte interphase (CEI) stabilizer

# The SEI is a passivation layer formed on the anode that regulates sodium-ion transport and prevents continuous electrolyte decomposition, while the CEI stabilizes cathode surface reactions and mitigates structural degradation.

# 6. Pouch Cell Mechanical Architecture (Stacked Design)

# The full cell follows a stacked pouch configuration consistent with sodium-ion prototype manufacturing systems:

- Form factor: stacked Z-fold pouch cell architecture

- Nominal voltage: 3.0–3.2 volts

- Target capacity class: 10 ampere-hour design point

# Layer stack:

- Cathode current collector: aluminum foil (~15 micrometers)

- Anode current collector: copper foil (~10 micrometers)

- Separator: polyolefin trilayer membrane (~20 micrometers)

- External casing: aluminum laminated moisture barrier (~30–50 micrometers)

- Inner sealant: polypropylene-based sealing layer

### Base Model Validation Framework

### 1. Electrochemical–Thermal Driver Model

### The cell behavior is first resolved using a DFN-based electrochemical framework implemented in PyBaMM.

### This model captures the coupled evolution of:  state of charge (SOC) trajectory, state of health (SOH) degradation trajectory, heat generation rate Q(t)arising from reaction, ohmic, and polarization losses

### These quantities define the internal energetic state of the system under charge–discharge operation.

### 2. Thermal Field Model (Heat Transport)

### The generated heat Q(t)is propagated through the cell using a thermal partial differential equation formulation implemented in PyBaMM thermal modules.

### This produces the spatial–temporal temperature field:  T(x,t)for spatially resolved thermal analysis or T(t) in lumped approximation for reduced-order cases

### 3. Thermoelastic Strain-Based Structural Integrity Model

### The temperature field T(x,t)serves as the primary excitation for the mechanical response of the electrode–electrolyte–interphase system. The structural behavior is modeled as a reduced-order thermoelastic continuum implemented in FEniCSx.

### The continuum consists of electrode phases, binder matrix, electrolyte, and SEI/CEI interphases. The model focuses on deformation-producing strain evolution under coupled electrochemical–thermal loading.

### 3.1 Deformation-producing strain drivers

### Strain-inducing expansion in the continuum is governed by:

### thermal expansion driven by spatial–temporal temperature gradients T(x,t)generated by:  reaction (entropic) heat, ohmic (Joule) heating in electrodes and electrolyte, charge-transfer (activation) losses, concentration polarization losses

### SOC-driven expansion from reversible insertion/deinsertion
SOH-driven drift in stiffness and expansion response

### These mechanisms collectively define deformation loading across all phases of the continuum.

### 3.2 Structural failure criterion (critical strain envelope)

### Structural integrity is not evaluated over the full strain field. Instead, only the lower-bound failure condition is retained:

### failure is triggered at the critical strain threshold envelope
defined as the minimum strain level at which irreversible deformation initiates

### The model evaluates: εcrit=min⁡(strain distribution at damage initiation)

### Failure occurs when local deformation-producing strain exceeds this critical envelope in any continuum phase.

  - Structural response metric (cycle–time deformation mapping)

### The structural output is expressed as a reduced endurance response under strain loading:  strain intensity evolution εint(t), threshold cycle–time response under varying strain intensity levels ncrit, tcrit)=f(εint

### where: ncrit= cycles to onset of irreversible deformation, tcrit= time to onset under operating profile, εint= applied or induced strain intensity

### This defines the deformation endurance boundary of the continuum under coupled electrochemical–thermal loading.

### NFPP Cell Optimization

### Objective Definition

### A constrained multi-objective optimization is formulated to minimize cost while maximizing electrochemical performance:

### min θ C(θ), max θ {E(θ), P(θ), L(θ)}

### Constraints:  Energy density ≥ 140 Wh/kg, Power capability ≥ 1C, Cycle life ≥ 8000 cycles, N/P ratio ≥ 1.05, Diffusion time ≤ charge time, Effective conductivity ≥ σ_min

### 1. Unified Parameterization (Design Space)

### The design space consists of:

### Structural parameters (θₛ):
electrode thickness (L_c, L_a), porosity (ε_c, ε_a), tortuosity (τ), active material loading, particle size (r_p)

### Material parameters (θₘ):
NFPP fraction, conductive carbon fraction, electrolyte composition, additive/doping strategy

### 2. Sensitivity-Driven Physics Reduction

### Sensitivity analysis is applied to DFN-derived outputs to identify parameters governing charge–discharge dynamics, electrode–electrolyte coupling, and rate-limiting transport and interfacial behavior.

### 2.1 Geometry and Interface Sensitivity (PyBaMM)

### PyBaMM geometry and reaction kinetics modules quantify sensitivity of performance to:

- electrode thickness, porosity, and loading

- charge-transfer resistance and interfacial overpotential

- internal structural configuration parameters (e.g., porosity distribution, tortuosity-controlled transport pathways, and binder-network compliance characteristics) that influence strain redistribution and suppress deformation localization under thermal and SOC cycling

### 2.2 Hessian-Based Design Space Reduction and Optimal Solution Identification

### Outputs from PyBaMM sensitivity analysis are embedded into a second-order Hessian curvature operator of the DFN performance landscape, extended to include strain-response coupling.

### This step performs simultaneous:

- coupling resolution across electrochemical, thermal, and mechanical response pathways

- constraint sensitivity compression (performance + structural integrity constraints)

- curvature-based elimination of weak or non-influential directions in the design space

### The reduced design space is defined as:

### θ→θ*⊂θ

### The optimal parameter set θ*is obtained by solving the constrained curvature system defined on the reduced DFN–strain landscape

### 2.3. Literature-Constrained Material Feasibility (Electrolyte and Additive Closure)

### A literature-guided feasibility filter is applied:

- candidate electrolyte systems and additives are retrieved from DFN-consistent experimental literature

- filtering is applied using:

  - electrochemical stability window compatibility

  - interfacial film stability (SEI / CEI formation consistency)

  - ionic transport and conductivity constraints

  - interfacial strain accommodation capability under thermo-mechanical cycling (compatibility with deformation suppression requirements identified in θ*)

### This produces a feasible material set conditioned on θ*, ensuring consistency across electrochemical performance and structural deformation resilience.

### 3. Cost-Driven Feasibility Adjustment

### cost acts as a correction operator on the coupled solution space obtained from 2

### The preliminary optimal solution (θ*) is adjusted under cost influence:

- material–structure pairs are modified jointly (not independently)

- adjustments preserve DFN feasibility constraints

- cost acts as a perturbation field over the admissible solution manifold

### This yields a cost-corrected feasible solution set:

### θ* → θ_c*

### where:

- improvements in cost are bounded and typically near-zero or negative relative to baseline

- any cost increase is constrained to negligible manufacturing tolerance levels

- electrochemical feasibility is preserved throughout adjustment

### 5. Stability Validation (Physics Consistency Check)

### The final optimized configuration is validated using a coupled reduced-order physics framework with PyBaMM, evaluating electrochemical and thermal behavior under full operating stress conditions.

### The model tracks SOC during discharge operation and HOC evolution alongside thermal PDE response under peak current loading and transient demand profiles.

### Computed cell-level performance metrics include:  Energy capacity (kWh), Nominal voltage (V), Continuous current (A), Peak current (A), Charge time (h or min under rated C-rate), Power capability (kW or C-rate equivalent), Cycle life (cycles to end-of-life under defined SOH threshold)

### 6. Expected Outcome

| ### Metric | ### Baseline | ### Optimized |
| --- | --- | --- |
| ### Energy density | ### 140 Wh/kg | ### 155–165 Wh/kg |
| ### Cycle life | ### 5000 | ### 8000–9000 |

### BMS Design

### System Overview

### The Battery Management System (BMS) is designed as a model-based, constraint-driven control system for sodium-ion ESS operation under unstable grid conditions.

## Pack Model

## Electrical Specification Cell Configuration 16S1P

## Thermal Management Architecture

# Thermal regulation is achieved using a hybrid passive cooling system combining phase-change material (PCM) for transient heat absorption and fin-based heat dissipation for continuous thermal rejection. The system is integrated at the pack level to mitigate transient heat accumulation during high C-rate operation and grid-induced load fluctuations

# BMS CONTROL SYSTEM + ESS SIMSCAPE ELECTRICAL MODEL

# The ESS is a constrained nonlinear plant:Pcell={SOC,V1,V2,T,SOH,R0eff}

# Control is applied through current: u=Icmd

# 2. CELLS (PHYSICAL PLANT)

# State vector

# x=SOCV1V2TSOH

# Dynamics: SOC evolution (pure state, not controlled) dSOCdt=-IQneff

# Electrical dynamics

# Vt=OCV(SOC)-IR0eff-V1-V2
V1=-V1R1C1+IC1,V2=-V2R2C2+IC2

# Thermal dynamics CthT=I2R0eff-hA(T-Tamb)

# SOH evolution (aging state) SOH=-f(I,T,SOC)

# 3. C-RATE CONTROLLER (PRIMARY CONTROL BLOCK)

# Control objective C=IQn

# Track power request while respecting constraints: Icmd=Cref⋅Qn

# Constraint-limited control law Icmd=sat(Iref,Imin(SOC,SOH),Imax(T,SOH))

# 4. CELL EQUALIZER CONTROLLER (FOR PACK)

# Objective: Minimize SOC dispersion, min⁡i(SOCi-SOC)2

# Equalization law

# For cell i Ieq,i=k(SOCi-SOC)

# Control mechanism

# Two implementations:

# (A) Passive balancing: resistor bleed, no active energy transfer

# (B) Active balancing; DC/DC shuttle between cells, controlled switching network

# 5. THERMAL LIMITER (SECONDARY CONTROL)

# a constraint scaler Icmdth=Icmd⋅e-λ(T-Tsafe)+

# Hard cutoff: T>85∘C⇒I=0

# 6. GRID STRESS DERATING BLOCK

# Pure supervisory constraint modifier.

# Stress metric Dk=α∣ΔV∣+β∣Δf∣+γB

# Current scaling Icmdgrid=Icmde-μDk

# All constraints collapse into current arbitration: I=min⁡(IC-rate,Ithermal,Igrid,ISOH)